DAG2000 Series Access Gateway Feature List V2.0

| Specifications | DAG2000 Series Access Gateway |
| --- | --- |
| General Information |  |
| Product Type | SIP VOIP access gateway |
| Product Model | DAG2000-16/24/32FXS |
| Market Target | For carrier and enterprise-class devices with cost-effective solution |
| Performance | Stable high-definition voice quality |
| Release Date | 22/03/2012 |
| Current Software Version | V2.15.01.01 |
| Current Hardware Version | PCB 1 |
| LEDs Indicator |  |
| PWR | Indicating the status of system power |
| RUN | Indicating the running status of device |
| WAN | Indicating WAN port activity |
| LAN | Indicating PC&LAN port activity |
| FXS | Indicating the status of the respective FXS ports on the back |
| Link | Indicating the status of the network connection |
| Speed | Indicating network bandwidth, 10Mbps/100Mbps |
| Ethernet Interface |  |
| Number of interface | 1/4 * 10/100 BASE-TX (RJ-45) |
| Auto adapted | Supported |
| Automatic flip | Supported |
| FXS Interface |  |
| Number of Interfaces | 16/24/32 * FXS |
| Transmission distance | 1km |
| Charging mode | Anti-polar signal |
| Signals & Protocols |  |
| SIP Signaling |  |
| Protocols compliant | SIP V1.0/2.0 and RFC3261 |
| Authentication | Supported |
| SIP encryption | Supported |
| Voice & FAX |  |
| G.723.1 | Supported |
| G.729 | Supported |
| PCM A/U | Supported |
| DTMF mode | RFC2833, SIP INFO and INBAND |
| FAX over IP | T.38/ Pass-Through/Modem, baud rate:14.4kbps |
| Trunk Features |  |
| Call forwarding | Supported |
| Call hold | Supported |
| DND | Supported |
| Caller ID | Time-Based service |
| PPPoE | Supported |
| Busy Tone | Supported |
| PSTN & VoIP routing | Users can set up routing to go PSTN or IP |
| Function keys | Support 26 function keys |
| Network Features |  |
| Network protocol | TCP/UDP, HTTP, ARP/RARP, DNS, NTP, TFTP, TELNET and STUN |
| Static IP | Supported |
| DNS | Supported |
| NAT | STUN |
| Static and dynamic ARP | Supported |
| VLAN | Data, Voice, Management |
| Qos | Tos / DSCP to ensure voice quality, data, management information transmission |
| DHCP Server | Supported |
| DHCP Client | Supported |
| Routing & number manipulation | Maximum 32 |
| Security & Reliability |  |
| Admin management | Administrator authentication by username and password |
| Maintenance & Upgrade |  |
| Web configuration | Supported |
| Telnet configuration | Supported |
| Support languages | English and Chinese |
| Firmware upgrade | TFTP and Web |
| Syslog type | ERROR,NONE,DEBUG,NOTICE,WARNING |
| CDR | Output CDR by Syslog |
| Follow-up testing | Ping and trace on web |
| Compatibility |  |
| Tested platforms | Work well with softswitch providers such as Huawei soft X3000, ZTE ZXSS10, Cisco, Alcatel, Lucent, S12. |
| Mechanical |  |
| Dimension (W*D*H) (mm) | 16FXS:442*280*44, 24/32FXS:442*250*44 |
| Net weight | 16FXS:3.0kg, 24/32FXS:3.5kg |
| Powering |  |
| Power output | 220VAC, 0.5A |
| Grounding | GND < 1Ω |
| Max power consumption | 16FXS:48W 24/32FXS: 80/100W |
| Availability-Redundancy |  |
| Service life time | 20 Years |
| Lightning protection | Telephone interface protection against lightning, 10/700/4000V |
| Environment-EMC |  |
| Operational | 32～104F / 0～40°C |
| Storage | -4～176F /-20～80°C |
| Humidity | 10%~90%, no condensation |
| Fixed use of environment | EN 300 019: Class 3.1 |
| Storage environment | EN 300 019: Class 1.2 |
| Transport Environment | EN 300 019: Class 2.3 |
| Noise | EN 300 753: Work in data center |
| EMC | FCC Part 15 Class B; CE EMC directive 2004/108/EC: EN55022：2006+A1：2007， EN61000-3-2：2006 EN61000-3-3：1995+A1：2001+A2：2005，EN55024：1998+A1：2001+A2：2003 |
| Safety | CE LVD EN60950-1：2006 |
| Certifications | FCC and CE |